Supplementary Data

This supplementary document reports only the data tables used in the study: the modified IEEE 32-bus feeder data, the practical 60-bus feeder data and the invited demand-response capacities of the six data centers.

1. IEEE 32-Bus Benchmark Feeder Data

The following tables provide the distributed-generator, branch, and node data of the modified IEEE 32-bus benchmark feeder used in the study.

Table S1. Distributed-generator data of the modified IEEE 32-bus feeder

| Generator | Node | P (kW) | Q (kVAr) |
| --- | --- | --- | --- |
| 1 | 24 | 600 | 500 |
| 2 | 19 | 480 | 340 |
| 3 | 3 | 570 | 500 |
| 4 | 16 | 510 | 440 |
| 5 | 27 | 400 | 320 |
| 6 | 30 | 490 | 450 |
| 7 | 10 | 800 | 600 |
| 8 | 14 | 420 | 380 |

Note. These distributed-generator entries correspond to the benchmark network reported in the manuscript.

Table S2. Branch data of the modified IEEE 32-bus feeder

| Branch | From node | To node | r (ohm) | x (ohm) |
| --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 0.493 | 0.2511 |
| 2 | 2 | 3 | 0.366 | 0.1864 |
| 3 | 3 | 4 | 0.3811 | 0.1941 |
| 4 | 4 | 5 | 0.819 | 0.707 |
| 5 | 5 | 6 | 0.1872 | 0.6188 |
| 6 | 6 | 7 | 0.7114 | 0.2351 |
| 7 | 7 | 8 | 1.03 | 0.74 |
| 8 | 8 | 9 | 1.044 | 0.74 |
| 9 | 9 | 10 | 0.1966 | 0.065 |
| 10 | 10 | 11 | 0.3744 | 0.1238 |
| 11 | 11 | 12 | 1.468 | 1.155 |
| 12 | 12 | 13 | 0.5416 | 0.7129 |
| 13 | 13 | 14 | 5.91 | 5.26 |
| 14 | 14 | 15 | 7.463 | 5.45 |
| 15 | 15 | 16 | 3.289 | 4.721 |
| 16 | 16 | 17 | 7.32 | 5.74 |
| 17 | 1 | 18 | 0.164 | 0.1565 |
| 18 | 18 | 19 | 1.5042 | 1.3554 |
| 19 | 19 | 20 | 0.4095 | 0.4784 |
| 20 | 20 | 21 | 0.7089 | 0.9373 |
| 21 | 2 | 22 | 4.512 | 3.083 |
| 22 | 22 | 23 | 0.898 | 0.7091 |
| 23 | 23 | 24 | 0.896 | 0.711 |
| 24 | 5 | 25 | 0.203 | 0.1034 |
| 25 | 25 | 26 | 0.2842 | 0.1447 |
| 26 | 26 | 27 | 1.059 | 0.9337 |
| 27 | 27 | 28 | 0.8042 | 0.7006 |
| 28 | 28 | 29 | 0.5075 | 0.2585 |
| 29 | 29 | 30 | 0.9744 | 0.963 |
| 30 | 30 | 31 | 0.3105 | 0.3619 |
| 31 | 31 | 32 | 0.341 | 0.5302 |

Note. The branch table reports the feeder topology and line impedances of the benchmark system.

Table S3. Node data of the modified IEEE 32-bus feeder

| Node | P load (kW) | Q load (kVAr) |
| --- | --- | --- |
| 1 | 100 | 60 |
| 2 | 90 | 40 |
| 3 | 120 | 80 |
| 4 | 60 | 130 |
| 5 | 0 | 0 |
| 6 | 100 | 100 |
| 7 | 200 | 100 |
| 8 | 60 | 50 |
| 9 | 0 | 0 |
| 10 | 45 | 30 |
| 11 | 60 | 35 |
| 12 | 60 | 35 |
| 13 | 120 | 80 |
| 14 | 60 | 10 |
| 15 | 60 | 20 |
| 16 | 60 | 20 |
| 17 | 90 | 40 |
| 18 | 0 | 0 |
| 19 | 90 | 40 |
| 20 | 90 | 40 |
| 21 | 90 | 40 |
| 22 | 90 | 50 |
| 23 | 220 | 200 |
| 24 | 420 | 200 |
| 25 | 60 | 25 |
| 26 | 60 | 25 |
| 27 | 60 | 20 |
| 28 | 120 | 10 |
| 29 | 200 | 600 |
| 30 | 350 | 70 |
| 31 | 210 | 100 |
| 32 | 60 | 40 |

Note. The node table reports the active and reactive load data of the benchmark system.

2. Practical 60-Bus Feeder Data

The following tables report the effective practical 60-bus feeder data, including the adopted base values, branch-parameter scaling, PV / generator expansion, and PV-capacity data.

Table S4. PV-generator buses and installed PV capacities in the practical 60-bus feeder

| Generator | PV-generator bus | Installed PV capacity (kW) |
| --- | --- | --- |
| 1 | 1 | 1000 |
| 2 | 3 | 230 |
| 3 | 5 | 210 |
| 4 | 8 | 200 |
| 5 | 15 | 220 |
| 6 | 25 | 190 |
| 7 | 27 | 220 |
| 8 | 32 | 200 |
| 9 | 34 | 210 |
| 10 | 38 | 190 |
| 11 | 39 | 230 |
| 12 | 40 | 200 |
| 13 | 41 | 210 |
| 14 | 46 | 230 |
| 15 | 47 | 190 |
| 16 | 48 | 600 |
| 17 | 49 | 540 |
| 18 | 50 | 220 |
| 19 | 52 | 200 |

Note. The practical-feeder study expands the generator and cost matrices consistently with this PV-generator bus list and capacity vector.

Table S5. Bus matrix of the practical 60-bus feeder

| Node | Type | P load (MW) | Q load (MVAr) | G shunt | B shunt | Area | V | Theta | Base kV | Zone | V max | V min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 0 | 0 | 0 | 0 | 1 | 0.9614 | 0.0223 | 10 | 1 | 1.1 | 0.9 |
| 2 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9602 | 0.0224 | 10 | 1 | 1.1 | 0.9 |
| 3 | 1 | 0.1859 | 0.0648 | 0 | 0 | 1 | 0.974 | 0.0499 | 10 | 1 | 1.1 | 0.9 |
| 4 | 1 | 0.0474 | 0.0199 | 0 | 0 | 1 | 0.9593 | 0.0207 | 10 | 1 | 1.1 | 0.9 |
| 5 | 1 | 0.1739 | 0.0541 | 0 | 0 | 1 | 0.974 | 0.0498 | 10 | 1 | 1.1 | 0.9 |
| 6 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9696 | 0.0412 | 10 | 1 | 1.1 | 0.9 |
| 7 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9793 | 0.0581 | 10 | 1 | 1.1 | 0.9 |
| 8 | 1 | 0.0906 | 0.0348 | 0 | 0 | 1 | 1.011 | 0.1064 | 10 | 1 | 1.1 | 0.9 |
| 9 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9949 | 0.0826 | 10 | 1 | 1.1 | 0.9 |
| 10 | 1 | 0 | 0 | 0 | 0 | 1 | 0.946 | -0.0002 | 10 | 1 | 1.1 | 0.9 |
| 11 | 1 | 0 | 0 | 0 | 0 | 1 | 0.946 | -0.0002 | 10 | 1 | 1.1 | 0.9 |
| 12 | 1 | 0 | 0 | 0 | 0 | 1 | 0.946 | -0.0002 | 10 | 1 | 1.1 | 0.9 |
| 13 | 1 | 0 | 0 | 0 | 0 | 1 | 0.946 | -0.0002 | 10 | 1 | 1.1 | 0.9 |
| 14 | 1 | 0.0474 | 0.0197 | 0 | 0 | 1 | 0.946 | -0.0002 | 10 | 1 | 1.1 | 0.9 |
| 15 | 1 | 0.0835 | 0.0358 | 0 | 0 | 1 | 0.9416 | -0.0081 | 10 | 1 | 1.1 | 0.9 |
| 16 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9461 | 0 | 10 | 1 | 1.1 | 0.9 |
| 17 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9461 | 0 | 10 | 1 | 1.1 | 0.9 |
| 18 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9601 | 0.024 | 10 | 1 | 1.1 | 0.9 |
| 19 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9506 | 0.008 | 10 | 1 | 1.1 | 0.9 |
| 20 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9461 | 0 | 10 | 1 | 1.1 | 0.9 |
| 21 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9461 | 0 | 10 | 1 | 1.1 | 0.9 |
| 22 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9949 | 0.0826 | 10 | 1 | 1.1 | 0.9 |
| 23 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9609 | 0.0252 | 10 | 1 | 1.1 | 0.9 |
| 24 | 1 | 0.0081 | 0.0042 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 25 | 1 | 0.0433 | 0.0123 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 26 | 1 | 0.0081 | 0.0043 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 27 | 1 | 0.0099 | 0.0004 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 28 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 29 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 30 | 1 | 0.0194 | 0.0092 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 31 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 32 | 1 | 0.1231 | 0.0405 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 33 | 1 | 0.0081 | 0.0035 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 34 | 1 | 0.081 | 0.0297 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 35 | 1 | 0.0081 | 0.0031 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 36 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 37 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 38 | 1 | 0.116 | 0.0448 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 39 | 1 | 0.008 | 0.0039 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 40 | 1 | 0.1084 | 0.0295 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 41 | 1 | 0.0222 | 0.0069 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 42 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 43 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 44 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 45 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 46 | 1 | 0.0367 | 0.0147 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 47 | 1 | 0.1 | 0.0248 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 48 | 1 | 0.0204 | 0.0033 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 49 | 1 | 0.0209 | 0.0008 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 50 | 1 | 0.1067 | 0.0287 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 51 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 52 | 1 | 0.0984 | 0.0253 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 53 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 54 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 55 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 56 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 57 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 58 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 59 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |
| 60 | 1 | 0 | 0 | 0 | 0 | 1 | 0.9604 | 0.028 | 10 | 1 | 1.1 | 0.9 |

Note. The columns follow MATPOWER conventions but are renamed here into paper-style engineering labels for readability.

Table S6. Branch matrix of the practical 60-bus feeder

| From node | To node | r | x | b | Rate A | Rate B | Rate C | Ratio | Angle | Status | Angle min | Angle max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 0.1795 | 0.2745 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 17 | 16 | 0.1795 | 0.264 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 2 | 4 | 0.1835 | 0.267 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 2 | 18 | 0.1825 | 0.2645 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 18 | 19 | 0.1715 | 0.2585 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 19 | 20 | 0.157 | 0.27 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 20 | 21 | 0.1875 | 0.285 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 21 | 16 | 0.162 | 0.26 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 1 | 23 | 0.1825 | 0.2835 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 23 | 33 | 0.193 | 0.272 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 33 | 36 | 0.154 | 0.2715 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 36 | 54 | 0.1985 | 0.259 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 54 | 37 | 0.1515 | 0.2595 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 37 | 55 | 0.192 | 0.281 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 55 | 46 | 0.192 | 0.2635 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 46 | 29 | 0.1525 | 0.278 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 29 | 28 | 0.1775 | 0.297 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 28 | 50 | 0.197 | 0.2855 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 50 | 56 | 0.166 | 0.284 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 56 | 31 | 0.1905 | 0.298 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 31 | 51 | 0.18 | 0.289 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 51 | 57 | 0.1895 | 0.2805 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 57 | 58 | 0.19 | 0.2975 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 58 | 24 | 0.1525 | 0.253 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 52 | 57 | 0.164 | 0.2635 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 58 | 53 | 0.1825 | 0.2995 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 35 | 34 | 0.1745 | 0.2885 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 36 | 35 | 0.1985 | 0.274 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 42 | 59 | 0.1875 | 0.284 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 59 | 44 | 0.1785 | 0.271 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 44 | 40 | 0.165 | 0.269 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 40 | 32 | 0.163 | 0.2605 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 39 | 42 | 0.1945 | 0.269 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 44 | 45 | 0.1725 | 0.2515 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 45 | 43 | 0.191 | 0.2735 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 59 | 41 | 0.155 | 0.2665 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 60 | 49 | 0.193 | 0.299 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 29 | 48 | 0.1515 | 0.278 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 48 | 60 | 0.195 | 0.2925 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 60 | 47 | 0.195 | 0.2705 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 54 | 38 | 0.176 | 0.273 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 38 | 39 | 0.156 | 0.2915 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 39 | 26 | 0.159 | 0.2995 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 26 | 25 | 0.1855 | 0.276 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 55 | 27 | 0.1915 | 0.2965 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 56 | 30 | 0.1515 | 0.287 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 18 | 6 | 0.188 | 0.2785 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 6 | 7 | 0.198 | 0.2985 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 7 | 3 | 0.167 | 0.291 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 3 | 5 | 0.182 | 0.298 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 19 | 14 | 0.167 | 0.2825 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 14 | 13 | 0.161 | 0.269 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 13 | 11 | 0.1895 | 0.274 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 11 | 10 | 0.186 | 0.2955 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 22 | 9 | 0.164 | 0.2505 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 13 | 12 | 0.179 | 0.258 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 20 | 15 | 0.171 | 0.2735 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 7 | 22 | 0.1545 | 0.277 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |
| 22 | 8 | 0.151 | 0.253 | 0 | 999 | 999 | 999 | 0 | 0 | 1 | -360 | 360 |

Note. The reported branch parameters already include the adopted scaling of r, x, and b.

3. Invited DR Capacities of the Six Data Centers

The following table reports the invited demand-response capacities of the six data centers over the 24 time slots.

Table S7. Invited DR capacities of the six data centers

| Time slot | DC1 invited DR capacity (kW) | DC2 invited DR capacity (kW) | DC3 invited DR capacity (kW) | DC4 invited DR capacity (kW) | DC5 invited DR capacity (kW) | DC6 invited DR capacity (kW) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 40 | 0 | 0 | 0 |
| 2 | 0 | 0 | 40 | 0 | 0 | 0 |
| 3 | 0 | 0 | 40 | 0 | 0 | 0 |
| 4 | 0 | 0 | 40 | 0 | 0 | 0 |
| 5 | 0 | 0 | 40 | 0 | 0 | 0 |
| 6 | 0 | 0 | 40 | 0 | 0 | 0 |
| 7 | 0 | 0 | 40 | 0 | 0 | 0 |
| 8 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | 0 | 0 | 0 | 0 | 0 | 70 |
| 12 | 0 | 0 | 0 | 0 | 0 | 70 |
| 13 | 0 | 0 | 0 | 0 | 0 | 70 |
| 14 | 0 | 0 | 0 | 0 | 40 | 70 |
| 15 | 0 | 0 | 0 | 0 | 40 | 70 |
| 16 | 0 | 0 | 0 | 0 | 40 | 70 |
| 17 | 0 | 0 | 0 | 0 | 40 | 70 |
| 18 | 0 | 0 | 0 | 0 | 40 | 0 |
| 19 | 0 | 80 | 0 | 50 | 0 | 0 |
| 20 | 0 | 80 | 0 | 50 | 0 | 0 |
| 21 | 0 | 80 | 0 | 50 | 0 | 0 |
| 22 | 60 | 0 | 0 | 50 | 0 | 0 |
| 23 | 60 | 0 | 0 | 50 | 0 | 0 |
| 24 | 60 | 0 | 0 | 50 | 0 | 0 |

Note. The values are reported as the period-specific invited DR capacities used in the practical-feeder case.